PM845GV1-C – Albatron

Intel Pentium� 4 Processor (Willamette/Northwood)
Socket 478 with FSB 400/533 MHz
2 DDR200/266/333 Memory Sockets
2 channel AC 97 Audio
2 ATA100 Channels, up to 4 ATA 100 IDE Devices
4 USB 2.0/1.1 Ports (2 ports by optional cable)
Integrated 3D Graphics Engine


Processor

Socket 478 Intel Pentium�4 Processor (Willamette/Northwood)

FSB

400/533 MHz

Chipset

North Bridge: Intel 845GV
South Bridge: Intel ICH4
AC97 Audio Codec: Realtek ALC201
I/O Chip : Winbond W83627HF


Memory

2 * DDR Sockets
DDR200/266/333 NON-ECC DDR SDRAM up to 2GB


Expansion Slots

1 x ADD Card Slot
2 x PCI slots (PCI 2.2 compliant)


IDE Connectors

2 ATA100 channels, up to 4 ATA 100 IDE devices

Onboard I/O Connectors

1 x Floppy Connector
2 x USB 2.0/1.1 header (2 ports by optional cable)
1 x CD_IN header
1 x COM (serial) port header (Optional COM cable)
1 x S/PDIF out header (S/PDIF out cable optional)
1 x CPU fan header with fan rotation detection function
2 x System fan headers (1 fan rotation detection function)
1 x 10 pin system panel header (Intel spec)
1 x Front audio header (Intel spec)
1 x IrDA header
1 x Case Open detection header


I/O via Back Panel

PS/2 keyboard/mouse, 2 x USB(2.0/1.1),1 x VGA ,1 x Com(serial), 1 x Parallel, 1x Game/Line-in/Line-out/MIC

Power

20-pin ATX power connector, 4-pin ATX 12V power connector

BIOS Feature

3Mb Flash EEPROM
Award BIOS with ACPI, DMI2.0, PnP, WfM2.0, Green Suspend to RAM (S3), Suspend to Disk (S4) Wake on keyboard/mouse, Wake on LAN/RTC Timer


Hardware Monitoring

2 FAN sensors, CPU/System voltages and temperature monitoring

Special Feature

Zero Jumper Design
Watch Dog Timer (auto-reset system when it can not handle overclock configurations)
AGP Protection (The AGP Protection can ensure the AGP card voltage to be 1.5V, to protect the mainboard and the AGP card.)


Certifications

FCC, CE, BSMI

Form Factor

Micro-ATX (244mm x 230mm)