MB i945PL ASUS P5PL2 PCI-E, DDR2 +sb, LAN S775

  CPU --- Socket 775 for Intel Pentium D / Pentium 4 / Celeron Intel® EM64T / EIST / Hyper-Threading Technology New power design supports Intel® next generation 04B/04A & 05B/05A CPU

  Chipset Intel --- 945PL Intel ICH7 Front Side Bus --- 800/533 MHz Memory --- 4 x DIMM, max. 2GB, DDR2 533/400, Non-ECC, un-buffered memory

  Dual Channel Architecture Expansion Slots --- 1 xPCI-E x16 2 xPCI-E x1 3 xPCI Storage/RAID --- Intel ICH7

  Southbridge: 1 x UltraDMA 100/66/33 4 x Serial ATA II (3Gb/s) Audio --- ADI1986A 6-ch Audio CODEC Jack Sensing and Enumeration S/PDIF out interface LAN --- RTL8111B PCI-E Gb LAN controller

  Overclocking Features --- SFS (Stepless Frequency Selection): from 100MHz up to 450MHz at 1MHz increment ASUS PEG Link " vCore: Adjustable CPU voltage at 0.025 V increment " ASUS EZFlash Adjustable FSB/DDR ratio. Fixed PCI/PCIe frequencies. 4-step DRAM voltage control ASUS C.P.R.(CPU Parameter Recall)

  Other ASUS Special Features ASUS Q-Fan ASUS MyLogo Back Panel I/O Ports --- 1 x Parallel 1 x Serial 1 x PS/2 Keyboard 1 x PS/2 Mouse 1 x Audio I/O 1 x RJ45 1 x S/PDIF Out (Coaxial+Optical) 4 x USB Internal I/O Connectors --- 2 x USB connector supports additional 4 USB ports CPU / Chassisx2 / Power FAN connectors 24-pin ATX Power connector 4-pin ATX 12V Power connector IDE LED connector, power LED connector Chassis Intrusion CD audio in GAME/MIDI connector Front panel audio connector BIOS 4 Mb Flash ROM, AMI BIOS, PnP, DMI2.0, WfM2.0, SM BIOS 2.3, ASUS EZ Flash, ASUS CrashFree BIOS 2, ASUS MyLogo

  Manageability WOL by PME,WOR by PME, Chassis Intrusion Support CD Drivers ASUS PC Probe II ASUS LiveUpdate Anti-Virus Software

  Accessories User's manual 1 x UltraDMA 133/100/66 cable 1 x USB 2-port Module 1 x FDD cable 1 x I/O shield 1 x SATA Power cables for 2 SATA Devices 2 x SATA cable

  Form Factor ATX 12"x 7.5" (30.5 cm x 19 cm) USB 8 USB2.0 ports