Shuttle MK35V/N

Socket A AMD Athlon XP, Athlon, Duron with 200/266 MHz FSB VIA KM266 + VT8235 Chipsets DDR 200/266 SDRAM, 2 x 184- pin DIMM, Up to 2 GB 2 x ATA 133 4X AGP device, 2x PCI slots, and 1 x CNR slot Onboard AC”97 two channel audio Onboard S3 Pro Savage 8 2D/3D Graphics 6x USB 2.0 Onboard VIA VT6103 10/100 Fast Ethernet (MK35 VN only) Micro ATX Form Factor.

### CPU Support List MK35V/N

| FSB | Multiplier | Processor |
| --- | --- | --- |
| 100MHz | 13x | AMD Duron(Morgan) 1.30 GHz |
| 100MHz | 12x | AMD Duron(Morgan) 1.20 GHz |
| 100MHz | 11x | AMD Duron(Morgan) 1.10 GHz |
| 100MHz | 10x | AMD Duron(Morgan) 1.00 GHz |
| 100MHz | 9.5x | AMD Duron 950 MHz |
| 100MHz | 9x | AMD Duron 900 MHz |
| 100MHz | 8.5x | AMD Duron 850 MHz |
| 100MHz | 8x | AMD Duron 800 MHz |
| 100MHz | 7.5x | AMD Duron 750 MHz |
| 100MHz | 7x | AMD Duron 700 MHz |
| 100MHz | 6.5x | AMD Duron 650 MHz |
| 100MHz | 6x | AMD Duron 600 MHz |
| 100MHz | 13x | AMD Duron (Palomino) 1.30 GHz |
| 133MHz | 13.5x | AMD Duron (Applebred) 1.80 GHz |
| 133MHz | 12x | AMD Duron (Applebred) 1.60 GHz |
| 133MHz | 10.5x | AMD Duron (Applebred) 1.40 GHz |
| 133MHz | 16x | AMD Athlon XP 2600+ |
| 133MHz | 15x | AMD Athlon XP 2400+ |
| 133MHz | 13.5x | AMD Athlon XP 2200+ |
| 133MHz | 13x | AMD Athlon XP 2100+ |
| 133MHz | 12.5x | AMD Athlon XP 2000+ |
| 133MHz | 12x | AMD Athlon XP 1900+ |
| 133MHz | 11.5x | AMD Athlon XP 1800+ |
| 133MHz | 11x | AMD Athlon XP 1700+ |
| 133MHz | 10.5x | AMD Athlon XP 1600+ |
| 133MHz | 10x | AMD Athlon XP 1500+ |
| 133MHz | 15x | AMD Athlon XP (Thorton) 2400+ |
| 133MHz | 13.5x | AMD Athlon XP (Thorton) 2200+ |
| 133MHz | 12.5x | AMD Athlon XP (Thorton) 2000+ |
| 100MHz | 9.5x | AMD Athlon 950 MHz |
| 100MHz | 9x | AMD Athlon 900 MHz |
| 100MHz | 8.5x | AMD Athlon 850 MHz |
| 100MHz | 8x | AMD Athlon 800 MHz |
| 100MHz | 7.5x | AMD Athlon 750 MHz |
| 100MHz | 7x | AMD Athlon 700 MHz |
| 100MHz | 6.5x | AMD Athlon 650 MHz |
| 133MHz | 10.5x | AMD Athlon 1.40 GHz |
| 100MHz | 14x | AMD Athlon 1.40 GHz |
| 133MHz | 10x | AMD Athlon 1.33 GHz |
| 100MHz | 13x | AMD Athlon 1.30 GHz |
| 133MHz | 9x | AMD Athlon 1.20 GHz |
| 100MHz | 12x | AMD Athlon 1.20 GHz |
| 133MHz | 8.5x | AMD Athlon 1.13 GHz |
| 100MHz | 11x | AMD Athlon 1.10GHz |
| 133MHz | 7.5x | AMD Athlon 1.00 GHz |
| 100MHz | 10x | AMD Athlon 1.00 GHz |
| Remark: 1. CPU internal core bus speed runs at 200/266MHz. |  |  |