Intel Pentium 4 1.7 GHz - RK80532PC029512

| General information |  |
| --- | --- |
| Type | CPU / Microprocessor |
| Family | Intel Pentium 4 |
| CPU part number | RK80532PC029512 |
| Frequency (MHz) ? | 1700 |
| Bus speed (MHz) ? | 400 |
| Clock multiplier ? | 17 |
| Package | 478-pin Flip-Chip Pin Grid Array (FC-PGA2) |
| Socket | Socket 478 (mPGA478B) |
| Architecture / Microarchitecture |  |
| Processor core | Northwood |
| Manufacturing process | 0.13 micron CMOS technology 55 million transistors |
| Data width | 32 bit |
| Floating Point Unit | Integrated |
| Level 1 cache size ? | Execution trace cache for up to 12K decoded micro-operations 8 KB data cache |
| Level 2 cache size ? | 512 KB 8-way associative full-speed on-die cache |
| Features | - MMX - SSE - SSE2 |