AMD Sempron 2500+ - SDA2500DUT3D

| Type | CPU / Microprocessor |
| --- | --- |
| Family | AMD Sempron |
| CPU part number | SDA2500DUT3D |
| Box part number | SDA2500BOX |
| Stepping codes | BCXJB BIXJB |
| Frequency (MHz) ? | 2500 (rated) 1750 (actual) |
| Bus speed (MHz) ? | 333 |
| Clock multiplier ? | 10.5 |
| Package | 453-pin organic PGA 1.95" x 1.95" (4.95 cm x 4.95 cm) |
| Socket | Socket A (Socket 462) |
| Introduction date | July 28, 2004 |
| Price at introduction | $74 |
| Architecture / Microarchitecture |  |
| Processor core | Thoroughbred (Model 8) |
| Manufacturing process | 0.13 micron |
| Floating Point Unit | Integrated |
| Level 1 cache size ? | 64 KB code 64 KB data |
| Level 2 cache size ? | Full speed on-die exclusive 256 KB |
| Features | - MMX technology - SSE - 3DNow! Professional |